Draft ETSI EN 303 347 V0.2.7 (2016-11)

Part 2: Harmonized Standard covering the essential requirements of article 3.2 of the Directive 2014/53/EU for C band Meteorological Radar Sensor operating in the frequency band 5 250 – 5 850 MHz

EUROPeAN STANDARD

Reference

DEN/ERM-TGAERO-28

Keywords

Radar, Radio

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the only prevailing document is the print of the Portable Document Format (PDF) version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 2015.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM and LTE™ are Trade Marks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

Reproduction is only permitted for the purpose of standardization work undertaken within ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

# Contents

Contents	4

Intellectual Property Rights	6

Foreword	6

Modal verbs terminology	6

Executive summary	7

1	Scope	8

2	References	8

2.1	Normative references	9

2.2	Informative references	9

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	11

3.3	Abbreviations	11

4	Technical requirements specifications	12

4.1	Environmental profile	12

4.2	Conformance  requirements	12

4.2.1	Transmitter requirements	12

4.2.1.1	Frequency Tolerance	12

4.2.1.1.1	Definition	12

4.2.1.1.2	Limits	12

4.2.1.1.3	Conformance	12

4.2.1.2	Out-of-Band emissions	12

4.2.1.2.1	Definition	12

4.2.1.2.2	Limits	14

4.2.1.2.3	Conformance	15

4.2.1.3	Spurious emissions	15

4.2.1.3.1	Definition	15

4.2.1.3.2	Limits	16

4.2.1.3.3	Conformance	16

4.2.2	Receiver Requirements	17

4.2.2.1	Receiver selectivity	17

4.2.2.1.1	Definition	17

4.2.2.1.2	Limit	17

4.2.2.1.3	Conformance	18

5	Testing for compliance with technical requirements	18

5.1	General requirements	18

5.2	Environmental conditions for testing	18

5.2.1	Normal temperature and humidity	19

5.2.2	Normal test power supply	19

5.3	Interpretation of the measurements results	19

5.4	Radio test suites	19

5.4.1	Transmitter test specification	19

5.4.1.1	Frequency Tolerance	19

5.4.1.2	Transmitter Power	20

5.4.1.3	Out-of-Band emissions	21

5.4.1.4	Spurious emissions	22

5.4.2	Receiver Test specification	23

5.4.2.1	Receiver Selectivity	23

5.4.2.1.1	General	23

5.4.2.1.2	Receiver OoB selectivity and spurious rejection	24

Annex A (normative):  Relationship between the present document and the essential requirements of Directive 2014/53/EU	25

Annex B (normative): Operating frequency, transmitter power and OoB measurement setup	26

Annex C (normative): Spurious emission measurement setup	27

Annex D (normative):  Receiver selectivity measurement setup	28

Change history	29

History	30

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This final draft Harmonised European Standard (EN) has been produced by ETSI Technical Committee Electric Magnetic Compatibility and Radio Spectrum Matters (ERM) and is now submitted for the combined Public Enquiry and Vote phase of the ETSI standards EN Approval Procedure.

The present document has been produced to provide a means of conforming to the essential requirements of Directive 2014/53/EU [i.1] of the European Parliament and of the Council of 16 April 2014 on the harmonisation of the laws of the Member States relating to the making available on the market of radio equipment  - also known as the Radio Equipment Directive1999/5/EC.

Once the present document is cited in the Official Journal of the European Union under that Directive, compliance with the normative clauses of the present document given in Table A.1 confers, within the limits of the scope of the present document, a presumption of conformity with the corresponding essential requirements of that Directive, and associated EFTA regulations.

The requirements relevant to Directive 2014/53/EU [i.1] are summarised in annex A.

The present document is part 2 of a multi-part deliverable covering meteorological radar systems for different frequency bands, as identified below:

Part 1:	„Harmonized Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU, Part 1 S-band meteorological systems radar in the frequency band 2 700 MHz to 3 100 MHz“

Part 2:	„Harmonized Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU, Part 2 C-band meteorological radar systems in the frequency band 5 250 MHz to 5 850 MHz“

Part 3:	„Harmonized Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU, Part 3 X-band meteorological radar systems in the frequency band 9 300 MHz to 9 500 MHz“

| Proposed national transposition dates |  |
| --- | --- |
| Date of latest announcement of this EN (doa): | 3 months after ETSI publication |
| Date of latest publication of new National Standard or endorsement of this EN (dop/e): | 6 months after doa |
| Date of withdrawal of any conflicting National Standard (dow): | 6 months after doa |

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "may not", "need", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Executive summary

The present document covers the essential requirements for efficient use of radio spectrum used by meteorological radar systems in the band 5 250 MHz – 5 850 MHz using pulsed signals. FM-CW or pulse compression signals will be covered in a later deliverable. The current version includes necessary changes due to adaption to the new Radio Equipment Directive 2014/53/EU [i.1].

# Scope

The present document is applicable to C-band meteorological radar systems intended for the surveillance and classification of hydrometeors with the following characteristics:

Operating in the following frequency range:

5 250 MHz to 5 850 MHz

Utilizing unmodulated pulses or phase/frequency modulated pulses also known as pulse compression.

The transceiver antenna connection and its feeding RF line are using a hollow metallic rectangular or elliptic waveguide

The antenna is rotating and can be changed in elevation.

The antenna feed is waveguide based and the antenna is passive.

The orientation of the transmitted field from the antenna can be vertical or horizontal orientated or it can be both simultaneously.

At the transceiver output a RF circulator is used.

NOTE 1:	Since transceiver and antenna are based on hollow metallic rectangular waveguide the frequency range for measurements that needs to be addressed covers 3 152 MHz to 26 GHz. The lower limit of this frequency range is obtained as the cut-off frequency of the generally used WR187/R48 waveguide according to IEC 60153-2 [i.2]. The upper limit corresponds to the upper limit stated in ERC/Recommendation 74-01 Table 1 [1].

NOTE 2:	Since at the transceiver output a RF circulator is used, it is assumed that the transceiver characteristics remain independent from the antenna.

NOTE 3:	Meteorological radar systems covered by the present document are expected to use the band 5 250 MHz to 5 850 MHz. According to provision 5.452 of the ITU Radio Regulations [i.4], ground-based radars used for meteorological purposes in the band 5 600 MHZ to 5 650 MHz are authorized to operate on a basis of equality with stations of the maritime radionavigation service.

NOTE 4:	Further technical and operational characteristics of meteorological radar systems can be found in[i.3].

The present document contains requirements to demonstrate that "... Radio equipment shall be so constructed that it both effectively uses and supports the efficient use of radio spectrum in order to avoid harmful interference", Directive 2014/53/EU [i.1].

In addition to the present document, other ENs that specify technical requirements in respect of essential requirements under other parts of article 3 of the Radio Equipment Directive 2014/53/EU [i.1] may apply to equipment within the scope of the present document.

# References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents that are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

## Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference/.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	ERC/Recommendation 74-01 (2011): "Unwanted emissions in the spurious domain".

[2]	ECC/Recommendation (02)05 (2012): "Unwanted emissions".

## Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	Directive 2014/53/EU of the European Parliament and of the Council of 16 April 2014 on the harmonisation of the laws of the Member States relating to the making available on the market of radio equipment and repealing Directive 1999/5/EC.

[i.2]	IEC 60153-2 (Edition 2.0, 1974): "Hollow metallic waveguides. Part 2: Relevant specifications for ordinary rectangular waveguides".

[i.3]	Recommendation ITU-R M.1849-1 (09/1015): “Technical and operational aspects of ground-based meteorological radars”

[i.4]	ITU Radio Regulations (2012).

[i.5]	ETSI TR 100 028 (all parts) (V1.4.1): " Electromagnetic compatibility and Radio spectrum Matters (ERM); Uncertainties in the measurement of mobile radio equipment characteristics".

[i.6]	ETSI TR 100 028-2 (V1.4.1): "Electromagnetic compatibility and Radio spectrum Matters (ERM); Uncertainties in the measurement of mobile radio equipment characteristics; Part 2".

[i.7]	Recommendation ITU-R M.1177-4 (04/2011): "Techniques for measurement of unwanted emissions of radar systems".

[i.8]	Recommendation ITU-R SM.1541-6 (09/2013): "Unwanted emissions in the out-of-band domain".

# Definitions, symbols and abbreviations

## Definitions

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

assigned frequency band: the frequency band within which the emission of a station is authorized; the width of the band equals the necessary bandwidth plus twice the absolute value of the frequency tolerance. Where space stations are concerned, the assigned frequency band includes twice the maximum Doppler shift that may occur in relation to any point of the Earth’s surface.

NOTE:	This definition is taken from the ITU Radio Regulations [i.4].

characteristic frequency: a frequency which can be easily identified and measured in a given emission. A carrier frequency may, for example, be designed as the characteristic frequency.

NOTE:	This definition is taken from the ITU Radio Regulations [i.4].

frequency tolerance: the maximum permissible departure by the centre frequency of the frequency band occupied by an emission from the assigned frequency or, by the characteristic frequency of an emission from the reference frequency. The frequency tolerance is expressed in parts in 106 or in Hertz.

NOTE:	This definition is taken from the ITU Radio Regulations [i.4].

necessary bandwidth BN: for a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions .

NOTE:	This definition is taken from the ITU Radio Regulations [i.4].

occupied bandwidth: the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage β/2 of the total mean power of a given emission.

NOTE 1: 	Unless otherwise specified in an ITU-R Recommendation for the appropriate class of emission, the value of β/2 should be taken as 0,5 %.

NOTE 2:	This definition is taken from the ITU Radio Regulations [i.4].

out-of-band emission: emission on a frequency or frequencies immediately outside the necessary bandwidth which results from the modulation process, but excluding spurious

NOTE:	This definition is taken from the ITU Radio Regulations [i.4].

peak envelope power (of a radio transmitter): the average power supplied to the antenna transmission line by a transmitter during one radio frequency cycle at the crest of the modulation envelope taken under normal operating conditions.

NOTE:	This definition is taken from the ITU Radio Regulations [i.4].

pulse duration: time in seconds between the 50 % amplitude (voltage) points of a transmitted pulse.

pulse rise time: time taken for the leading edge of the pulse to increase from 10 % to 90 % of the maximum amplitude (voltage) in seconds.

receiver selectivity: the ability of a receiver to detect and decode a desired signal in the presence of an unwanted interfering signal which is usually in the adjacent band.

reference frequency: a frequency having a fixed and specified position with respect to the assigned frequency. The displacement of this frequency with respect to the assigned frequency has the same absolute value and sign that the displacement of the characteristic frequency has with respect to the centre of the frequency band occupied by the emission.

NOTE:	This definition is taken from the ITU Radio Regulations [i.4].

spurious emission: emission on a frequency or frequencies which are outside the necessary bandwidth and the level of which may be reduced without affecting the corresponding transmission of information. Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products, but exclude out-of-band emissions.

NOTE:	This definition is taken from the ITU Radio Regulations [i.4].

system coupler: a high power directional waveguide coupler with forward and reverse port or only a forward port. The system coupler is inserted in the waveguide run between the circulator and the antenna but not directly located behind the antenna. Usually it is located very close behind the circulator.

transmitter coupler: a high power directional waveguide coupler with forward and reverse port or only a forward port. The transmitter coupler is inserted in the waveguide run between the output of the transmitter and the power divider used for dual polarisation mode or the output of the transmitter and the first circulator. Usually it is located very close behind the transmitter output. It is also usually the first coupler in a radar system waveguide run.

## Symbols

For the purposes of the present document, the following symbols apply:

B-40	-40 dB bandwidth

BN	Necessary bandwidth

dB/dec	dB per decade

dBpp	dB with respect to peak power

fc	characteristic frequency

ft	transmitter frequency tolerance

t	Pulse duration

tr	Pulse rise time

## Abbreviations

For the purposes of the present document, the following abbreviations apply:

AC	Alternating Current

A/D	Analog to digital converter

CW	Continuous Wave

EFTA	European Free Trade Association

FM	Frequency Modulation

IF 	Intermediate frequency

LNFE	Low Noise Front End

MDS	Minimmum detectable signal

OoB	Out of Band

PEP	Peak Envelope Power

PM	Phase Modulation

PRF	Pulse Repetition Frequency

RF	Radio Frequency

# Technical requirements specifications

## Environmental profile

The technical requirements of the present document apply under the environmental profile for operation of the equipment, which shall be declared by the manufacturer. The equipment shall comply with all the technical requirements of the present document at all times when operating within the boundary limits of the declared operational environmental profile.

## Conformance  requirements

### Transmitter requirements

#### Frequency Tolerance

##### Definition

The transmitter of a pulsed radar system produces microwave pulses which cause a broad frequency spectrum depending on the pulse duration and transmitter. The operating frequency is the frequency of the microwave emission during the transmitting pulse and is represented by the spectral line of highest amplitude. For phase/frequency modulated radar systems the operating frequency is to be understood as the centre between the highest and lowest transmitted frequency.

NOTE:	It is only practicable to indicate a single operating frequency for radars with unmodulated pulses. In this case a limit for the frequency tolerance is specified.

##### Limits

The frequency tolerance for meteorological radar systems applying unmodulated pulses shall be:

ft=1 250∙10-6∙fc	(1)

For radars with modulated pulses above specified frequency tolerance shall apply to all frequencies used in the modulation.

For all radar types covered by the present documents the occupied bandwidth of the signal shall be contained completely within the frequency ranges 5 250 MHz to 5 850 MHz in all operating modes.

##### Conformance

The conformance tests are specified in clause 5.4.1.1.

#### Out-of-Band emissions

##### Definition

An important parameter of the Out-of-Band (OoB) emissions mask of the radar is the -40 dB bandwidth. Annex 8 of Recommendation ITU R SM.1541 6 [i.8] specifies the -40 dB bandwidth specified for various types of waveforms (e.g. pulsed radar signals).

The -40 dB bandwidth (B-40) for non-FM/PM pulse radars is the lesser of:

B-40=Kt ∙ tr  or 64t 	(2)

Where:

the coefficient K is 6.2 for meteorological radar systems with operating power greater than 100 kW and 7.6 for lower-power radars.

t is the pulse duration between the 50% amplitude (voltage) points in seconds.

tr is the rise time in the case of a trapezoidal pulse.

NOTE 1:	For typical values of a pulse duration of t = 500 ns and a rise time of tr = 100 ns with a PEP of 250 kW the formula above yields a 40 dB bandwidth value of 27,7 MHz.

For frequency modulated pulse radar systems the -40 dB bandwidth is:

B-40=1,5BC+π∙lnBC∙τ0,53 ∙MinBrise,Bfall, Brise&fall+MaxBrise,Bfall, Brise&fall	(3)

Where:

BC is the bandwidth of the frequency deviation (total frequency shift during the pulse generation).

τ is the pulse length including rise and fall times.

Brise=1τ∙tr to account for the rise time.	(4)

Bfall=1τ∙tf to account for the fall time.	(5)

Brise&fall=13τ∙tr∙tf to account for both the rise and fall times combination.

tr is the rise time.

tf is the fall time.

The equation 3 above is only valid when the following conditions are met:

The product BC ∙ Minimum (tr, tf) is greater than or equal to 0,10 and

that the product of BC ∙ τor compression ratio must be freater than 10.

In all other cases, the following equations should be used:

B-40=Kt ∙ tr +2BC+Atr	(6)

Where:

A is 0.105 when K = 6,2 and 0,065 when K 7,6.

- For FM pulse radar with frequency hopping, the value of BS needs to be added to the value of B-40 equation 3 or 6 for the frequency hopping radar B-40 bandwidth where BS is the maximum range over which the carrier will be shifted, BS equals zero for non-frequency hopping cases.

Note 2:	The term A/tr adjusts the value of B−40 to account for the influence of the rise time, which is substantial when the time-bandwidth product Bc ∙ t, is small or moderate and the rise time is short.

Note 3:	This yields the total composite B−40 bandwidth of frequency hopping radar as if all channels included within Bs were operating simultaneously. For frequency hopping radars, the OoB emission mask falls off from the edge of the B−40 dB bandwidth as though the radar were a single frequency radar tuned to the edge of the frequency hopping range.

For radars with multiple pulse waveforms, the B-40 bandwidth shall be calculated for each individual pulse length and the maximum B-40 bandwidth obtained shall be used to establish the shape of the emission mask.

For radars with an asymmetrical spectrum, the B-40 dB bandwidth can be offset from the frequency of maximum emission level, but the B-40  bandwidth shall be contained completely within the allocated band as stipulated in section 4 of Annex 8 of Recommendation ITU R SM.1541-6 [i.8].

The application of this rule is illustrated in Figure 1.

Figure 1: Application of the offset-rule for the OoB emission limit mask

##### Limits

The maximum OoB emission power level shall not exceed the limits stated in Table 1 or Table 2 and shall not exceed the corresponding mask depicted in Figure 2. The roll-off of the OoB mask beyond the -40 dB bandwidth in relation to B-40 is specified as follows:

The mask has a roll-off at 30 dB/dec from the calculated (identified) B-40 bandwidth to a level of -70 dBpp.

The mask then continues to roll-off at 60 dB/dec to a spurious emission limit level of -100 dBpp or -90 dBpp with regard to the PEP. Further details to the level can be found in chapter Limits.

NOTE 2:	The -100 dBpp mask corresponds to the dashed line in figure A2.1c and the -90 dBpp corresponds to the figure A2.1b of unwanted emissions in Annex 2 of the ECC/Recommendation (02)05 [2].

NOTE 3:	ERC/Recommendation 74-01 [1] stipulates in its Table 5.1 for fixed radars a spurious emission limit in the reference bandwidth of "-30 dBm or 100 dB/90 dB, whichever is less stringent".

Table 1: Limits for unwanted emissions with a PEP of greater than 150 kW

| Frequency offset relative to B-40 | Limit dBpp | Slope dB/decade |
| --- | --- | --- |
| 0 to 0,5 | 0 | 0 |
| 0,5 | -40 | - |
| 0,5 to 5 | -40 to -70 | -30 |
| 5 to 10,8 | -70 to -90 or -30 dBm (see note 1) | -60 |
| 10,8 to | -90 or -30 dBm (see note 2) | 0 |
| NOTE 1: -70 to -90 or -30 dBm whichever is less stringent. NOTE 2: -30 dBm or -90 dB, whichever is less stringent. |  |  |

Table 2: Limits for unwanted emissions with a PEP of equal or lower than 150 kW

| Frequency offset relative to B-40 | Limit dBpp | Slope dB/decade |
| --- | --- | --- |
| 0 to 0,5 | 0 | 0 |
| 0,5 | -40 | - |
| 0,5 to 5 | -40 to -70 | -30 |
| 5 to 15,8 | -70 to -100 or -30 dBm (See note 1) | -60 |
| 15,8 to | -100 or -30 dBm (see note 2) | 0 |
| NOTE 1: -70 to -100 or -30 dBm whichever is less stringent. NOTE 2: -30 dBm or -100 dB, whichever is less stringent. |  |  |

Figure 2: OoB emission limit masks

##### Conformance

The conformance tests are specified in clause 5.4.1.3.

#### Spurious emissions

##### Definition

Spurious emissions are defined as the entity of all emissions in the frequency range from the cut-off frequency 3 152 MHz of the waveguide section to 26 GHz, but outside B-40 boundaries and outside the  OoB domain..

NOTE:	The lower limit of this frequency range is obtained as the cut-off frequency of the generally used WR187/R48 waveguide according to IEC 60153-2 [i.2]. The upper limit corresponds to the upper limit stated in ERC/Recommendation 74-01 [1] Table 1.

They include:

harmonic emissions (whole multiples of the operating frequency)

parasitic emissions (independent, accidental)

intermodulation (between oscillator- and operation frequency or between oscillator and harmonics)

emissions caused by frequency conversions

The boundaries between OoB domain and the spurious domain are where the OoB limit mask specified in ECC/Recommendation (02)05 [2] reaches the spurious emission limit of -100 dBpp or -90 dBpp according to ERC/Recommendation 74-01 [1] Table 5.1. This is illustrated in Figure 3.

Figure 3: Definition of OoB and spurious emission domains
(Not to scale).

##### Limits

For meteorological radar systems the spurious emission limits are related to the PEP. The limits shall be as specified in in table X.  These limits are from ERC/Recommendation 74-01 [1] Annex 5.

For the spurious emissions the following requirement based on Table 5.1 in annex 5 for the case of fixed stations in ERC/Recommendation 74-01 [1] shall apply:

Table 3: Spurious emission levels

| Transmitter PEP | Spurious emission limits |
| --- | --- |
| < 10 kW | -30 dBm |
| 10 kW < PEP ≤ 150 kW | 100 dB |
| > 150 kW | 90 dB |

The spurious emission limits are either absolute levels (dBm in PEP in the reference bandwidth) or attenuation (dB) below the PEP supplied to the antenna port.

NOTE:	In the case of occurrence of interferences caused by unwanted emissions of the radar transmitter, much higher suppression of Out-of-Band or spurious emissions may be required during measurement. Therefore, it is desirable that it is possible to attenuate or to suppress parts of the emitted signal in the feeder line to the measurement equipment.

##### Conformance

The conformance tests are specified in clause 5.4.1.4.

### Receiver Requirements

#### Receiver selectivity

##### Definition

The receiver selectivity is the ability of a receiver to detect and decode a desired signal in the presence of an unwanted interfering signal which is usually in the adjacent band.

##### Limit

The selectivity of the radar system shall increase in the same degree as the permitted emission spectrum with a limit of -90 dBpp decreases. The -90 dBpp curve corresponds to the dashed line in figure A2.1b of unwanted emissions in Annex 2 of the ECC/Recommendation (02)05 [2]. The maximum input power of the receiver shall not exceed -30 dBm. The MDS level equals the noise power at the output of the receiver when the input of the receiver is terminated with a 50 Ohm attenuator at room temperature.

Table 4: Receiver selectivity levels with regard to MDS level

| Frequency offset relative to B-40 | Maximum interfering power level dB above MDS | Slope dB/decade |
| --- | --- | --- |
| 0 to 0,5 | None | 0 |
| 0,5 | 40 | - |
| 0,5 to 5 | + 40 to 70 or -30 dBm (see note 1) | -30 |
| 5 to 10,8 | 70 to 90 or -30 dBm (see note 1) | -60 |
| 10,8 to | -30 dBm | 0 |
| NOTE 1: The maximum input power of the receiver shall not exceed -30 dBm |  |  |

The receiver selectivity shall be verified in the OoB and spurious domain covering the frequency range from 3 152 MHz to 26 GHz.

NOTE 2:	The lower limit of this frequency range is obtained as the cut-off frequency of the generally used WR187/R48 waveguide according to IEC 60153-2 [i.2]. The upper limit corresponds to the upper limit stated in ERC/Recommendation 74-01 [1] Table 1.

Figure 4: Receiver selectivity mask (Not to scale).

##### Conformance

The conformance tests are specified in clause 5.4.2.1.

# Testing for compliance with technical requirements

## General requirements

For the purpose of the compliance tests described in the present document, the radar under test shall be set up in a realistic operation mode. This means that the transceiver shall be operating and set-up with parameters which produce the worst-case spectrum (e.g. shortest pulse length, highest peak frequency deviation). Furthermore, the radar shall be supplied with all the necessary signals (e.g. antenna azimuth encoder signal, safety loop signals,) to simulate normal operation.

NOTE:	The standard operating parameters depend very much on the type of the radar. In the test report the mode of operation applied for the tests shall be documented.

## Environmental conditions for testing

5.2.1		Test Conditions

Tests defined in the present document shall be carried out at representative points within the boundary limits of the manufacturer declared operational environmental profile.

Where technical performance varies subject to environmental conditions, tests shall be carried out under a sufficient variety of environmental conditions (within the boundary limits of the manufacturer declared operational environmental profile) to give confidence of compliance for the affected technical requirements.

### Normal temperature and humidity

The normal temperature and humidity conditions for tests typically are a combination of temperature and humidity within the following ranges:

temperature: +15°C to +25°C

relative humidity: 20 % to 75 %

Actual values shall be stated in the test report.

### Normal test power supply

The test voltage for the equipment to be connected to an AC supply shall be the nominal mains voltage declared by the manufacturer including a variation of ±10 %. For the purpose of the present document, the nominal voltage shall be the declared voltage or each of the declared voltages for which the equipment is indicated as having been designed. The frequency of the test voltage shall be 50 Hz ± 1 Hz.

## Interpretation of the measurements results

The interpretation of the results recorded in a test report for the measurements described in the present document shall be as follows:

the measured value related to the corresponding limit will be used to decide whether an equipment meets the requirements of the present document

the value of the measurement uncertainty for the measurement of each parameter shall be included in the test report

the recorded value of the measurement uncertainty shall be, for each measurement, equal to or lower than the figures in Table 5

For the test methods, according to the present document, the measurement uncertainty figures shall be calculated and shall correspond to an expansion factor (coverage factor) k = 1,96 or k = 2 (which provide confidence levels of respectively 95 % and 95,45 % in the case where the distributions characterizing the actual measurement uncertainties are normal (Gaussian)). Principles for the calculation of measurement uncertainty are contained in ETSI TR 100 028 [i.5], in particular in annex D of the ETSI TR 100 028-2 [i.6].

Table 5: Maximum measurement uncertainty

| Parameter | Uncertainty |
| --- | --- |
| Transmitter measurements |  |
| Operating frequency | ±1∙10-5 |
| Out-of-Band emissions | ± 4 dB |
| Spurious emissions | ± 4 dB |
| Receiver measurements |  |
| Receiver Selectivity | ± 4 dB |

## Radio test suites

### Transmitter test specification

#### Frequency Tolerance

The antenna shall either be replaced by a suitable high power dummy load or pointing 90 degree upwards. The forward port of the system coupler shall be used and shall have an adequate attenuation. An optional reverse port shall be terminated with an appropriate 50 Ω terminator.

To measure the operating frequency a suitable frequency meter or spectrum analyser shall be used. The frequency meter shall be capable of measuring the short RF pulses. An additional attenuator shall be used if needed in order to protect the frequency meter input from the high power RF pulses. The measurement setup from Annex B shall be used.

The frequency measurements shall be performed with all available pulse length settings. The corresponding PRF shall be chosen in order to get the maximum possible duty cycle for each pulse length. After the frequencies for the maximum duty cycles are measured, the measurements shall be repeated with the lowest duty cycle. The lowest duty cycle is defined as the combination of shortest pulse length and lowest PRF. The lowest PRF shall be the one, which will be generally used in meteorological radar systems during normal operation.

NOTE:	A typical lower value for the PRF is 250 Hz as mentioned in [i.3].

Between each measurement, a waiting period of at least 20 minutes shall be applied. During this time, the transmitter shall be in operation and transmitting with the new pulse length and PRF values. This will give the transmitter enough time to adjust thermally. If the transmitter has not thermally adjusted or no thermal drift compensation the waiting period shall be extended until the frequency drift has come to an end.

For frequency or phase modulated radar systems the operating frequency is the centre frequency between the highest and lowest transmitted frequencies. Preferably a spectrum analyser shall be used to display the occupied frequency spectrum in order to obtain the centre between the highest and lowest frequencies.

The results obtained for all available pulse length settings shall be compared with the limits in clause 4.2.1.1.2 in order to prove compliance with the requirement.

To measure the frequency stability a frequency counter 	with a frequency stability of equal to or better than 10 ppm is connected to the radar transmitter via suitable couplers.

#### Transmitter Power

The transmitter output power is used for the calculation of the OoB and spurious emission mask. The power levels of the emission masks are relative to the peak power of the transmitter.

The antenna shall be replaced by a suitable high power dummy load or pointing 90 degree upwards. If the meteorological radar system is equipped with dual polarisation capability, the single polarisation mode shall be activated and shall be used for the measurements. If only permanent dual polarisation mode is available and no coupler in front of the power divider is available, the coupling ratio from the power divider shall be taken into account for. The forward port of the transmitter coupler shall be used and shall have an adequate attenuation. An optional reverse port shall be terminated with an appropriate 50 Ω terminator. The coupling factor shall be known in the allocated band with an accuracy of ± 0,5 dB or better. The measurement from Annex B shall be used.

The transmitter power of a pulse radar is considered to be the peak value of the transmitter pulse power during the transmission pulse (PEP).

If the transmitter power varies over the azimuth, the highest PEP value measured during a period equal to at least one rotation period shall be used.

The transmitter power measurements shall be performed with all available pulse length settings. The corresponding PRF shall be chosen in order to get the same duty cycle for each pulse length setting.

To determine the PEP of the pulse a suitable pulse power meter with direct reading of the transmitter pulse power shall be used. The PEP shall be measured at the 50% point of the pulse length. If the transmitter pulse is rippled the average over the pulse shall be used as can be seen in Figure 5.

Figure 5: Transmitter output power

To reference the indicated transmitter power to the transmitter output flange the coupling factor of the transmitter coupler shall be taken into account. If an additional attenuator or RF cable is installed between the transmitter coupler forward port and the power meter this shall be taken into account. If the power meter does not allow for compensation of the coupling loss and additional attenuator, then the coupling loss and attenuator value shall be added to the meter reading.

The results obtained shall be compared to the limits in clause 4.2.1.2.2 in order to prove compliance with the requirement.

#### Out-of-Band emissions

The antenna shall be replaced by a suitable high power dummy load or pointing 90 degree upwards. If the meteorological radar system is equipped with dual polarisation capability, the single polarisation mode shall be activated and shall be used for the measurements. If only permanent dual polarisation mode is available and no coupler in front of the power divider is available, the coupling ratio from the power divider shall be taken into account. The forward port of the system coupler shall be used and shall have an adequate attenuation. An optional reverse port shall be terminated with an appropriate 50 Ω terminator. The coupling factor shall be known in the allocated frequency band with an accuracy of ± 0,5 dB or better.

The measurement bandwidth shall be according to Recommendation ITU-R M.1177-4 [i.7].

The so-called indirect method shall be applied for the measurement of unwanted emissions of radar systems. The transmitter output spectrum shall be measured at the system coupler of the transmitter as illustrated in Annex B.

NOTE 1:	To obtain a sufficient dynamic range the radar signal need to be suppressed by an additional notch filter.

Further information how to perform the measurement can be found in Recommendation ITU-R M.1177-4 . The OoB power emission shall be measured in the frequency bands given in Table 6 or Table 7 depending on the PEP. The results obtained shall be compared to the limits in clause 4.2.1.3.2 and depicted in Figure 2 in order to prove compliance with the requirement.

NOTE 2:	The following OoB-boundaries are taken from ECC/Recommendation (02)05 [2].

Table 6: OoB emissions boundaries for -90 dBpp

| Lower OoB boundary | Upper OoB boundary |
| --- | --- |
| Carrier frequency – 10,8 × B-40 | Carrier frequency + 10,8 × B-40 |

Table 7: OoB emissions boundaries for -100 dBpp

| Lower OoB boundary | Upper OoB boundary |
| --- | --- |
| Carrier frequency – 15,8 × B-40 | Carrier frequency + 15,8 × B-40 |

NOTE 3:	Typical meteorological radar system parameters are e.g. a centre frequency of 5 640 MHz, transmitter power of 250 kW, a pulse duration of t = 500 ns and a rise time of tr = 100 ns. The 40 dB bandwidth calculated applying the equation from clause 4.2.1.3.1 is 27,7 MHz. This leads to OoB boundaries at 10,8 × 27,7 MHz = 299,2 MHz away from the operating frequency. For this example the absolute boundaries between OoB emissions and spurious emissions are: 5 640 MHz – 299,2 MHz = 5 340,8 MHz and 5 640 MHz + 299,2 MHz = 5 939,2 MHz (see Figure 6).

Figure 6 show the calculated emission masks for the aforementioned parameters of a typical meteorological radar system applying the mask specification in clause 4.2.1.3 which is corresponding to the dashed line in figure A2.1b of ECC/Recommendation (02)05 [2].

Figure 6: Calculated emissions mask for pulse duration t = 500 ns and rise time tr = 100 ns at centre frequency of 5 640 MHz

#### Spurious emissions

For the spurious emission measurements the aforementioned indirect method shall be used. To perform the measurements the radar system and the measuring equipment shall be set up as displayed in Annex C. The spurious power emissions shall be measured in the frequency ranges outside the OoB emissions boundaries.

NOTE: 	Depending on the setup of the meteorological radar system the location where the measurement setup will be installed may be close to the antenna. This ensures that band-limiting components like circulator, rotary joint or waveguide filter are included in the measurement.

Wave propagation in the waveguide is not possible below a certain cut-off frequency where the attenuation of the waveguide is very high. Beyond a certain upper frequency limit, several propagation modes are possible so that the behaviour of the waveguide is no longer unambiguous. In the unambiguous range of a rectangular waveguide, only TE1,0 waves are capable of propagation. In the WG16 waveguide the cut-off frequency is 6 556 MHz which is higher than the operating frequency of the C-Band meteorological radar systems. Therefore, at least a 15 cm long WG16 waveguide shall be inserted in the measurement setup in order to protect the measurement device from the operating frequency in the WG16 and higher waveguide bands. The waveguide acts as a high pass in this setup.

Due to the ambiguous propagation modes of the used C-Band waveguide for higher frequencies, smaller waveguides with appropriate linear tapers shall be used for the measurement of higher frequencies. These frequency ranges are also referred to as waveguide bands as can be seen in Table 8.

Each waveguide band shall be measured with its corresponding waveguide resulting in unambiguously measurements for the spurious measurements.

EXAMPLE:	For the measurement of the frequency range 8,2 – 12,4 GHz the following setup will be used: a taper from WG12 to WG14, followed by a second taper from WG14 to WG16 waveguide, followed by at least 15 cm of WG16 waveguide terminated with a WG16 to coax transition.

Table 8: Waveguide bands and associated waveguides

| Waveguide band | Frequency | Cut-off frequency | Waveguide designation |  |
| --- | --- | --- | --- | --- |
|  |  |  | EIA | UK |
| C | 3,95 – 5,85 GHz | 3,152 GHz | WR187 | WG12 |
|  | 5,85 – 8,2 GHz | 4,300 GHz | WR137 | WG14 |
| X | 8,2 – 12,4 GHz | 6,556 GHz | WR90 | WG16 |
| Ku | 12,4 – 18,0 GHz | 9,486 GHz | WR62 | WG18 |
| K | 18,0 – 26,5 GHz | 14,051 GHz | WR42 | WG20 |

A noise margin of at least 10 dB below the spurious emission levels of -100 dBpp or -90 dBpp shall be achieved. A notch filter for the operating frequency shall be used to achieve the required dynamic amplitude range.

NOTE:	In the taper from the WG14 to the WG16 waveguide the operating frequency will be completely reflected. If the connected circulator is the internal one and has not been installed purely for the measurement it will transfer the signal to the receiver input. Therefore the LNFE shall be replaced by a suitable high power dummy load.

The obtained results shall be compared to the limits in clause 4.2.1.4.2 in order to prove compliance with the requirement.

Table 9: Spurious emissions measurement bands

| Lower measurement band | Upper measurement band |
| --- | --- |
| From 3 152 MHz to the lower OoB boundary | From the upper OoB boundary to 26 GHz |

### Receiver Test specification

#### Receiver Selectivity

##### General

The radar receiver is setup in normal operating mode during the test. The receiver frequency should be tuned to the centre frequency of ground-based radars used for meteorological purposes which is at 5 625 MHz. If the meteorological radar system is operating outside the abovementioned range is shall be tested at the actual operating frequency.

Compliance shall be tested by subjecting the LNFE input directly, or in conjunction with its connecting waveguide to signals at discrete frequency steps within the spurious and OoB domain. Depending on the radar setup the waveguide components between the LNFE and the antenna may have bandwidth limiting functions and should be incorporated in the receiver selectivity measurement. The measurement setup from Annex D shall be used.

The LNFE input is defined as the coaxial input port, which is connected directly via a short RF cable to the waveguide-coax transition in normal operation of the radar system. The IF output of the LNFE is defined as the port which is connected directly via a RF cable to the A/D converter of the digital receiver on normal operation of the radar system. Both ports can be seen in Figure 7.

NOTE:	Usually the IF frequency prior the A/D converter is 60 MHz.

If the meteorological radar system has two independent receiving channels for each polarisation, the one with the highest sensitivity shall be chosen. If direct conversion receivers with I and Q mixer are used the selectivity shall be measured at both channels.

##### Receiver OoB selectivity and spurious rejection

Frequencies inside the B-40 bandwidth need not to be tested because this is the receiving frequency range of the meteorological radar system. No rejection of unwanted signals in the LNFE is possible in this frequency range. The LNFE output power shall be measured at the abovementioned centre or operating frequency in order to get a reference level for the evaluation of rejection levels in the OoB and spurious domain.

NOTE 1:	Due to a possible saturation of the LNFE at the operating frequency, it may be necessary to decrease the LNFE input power. The maximum LNFE input power shall be below its upper limit of the linear operation range to prevent saturation.

If the LNFE input power has been reduced the obtained results shall be corrected in relation to the LNFE input power.

The disturbance signals shall be applied either directly to the LNFE input or shall be applied to the connecting waveguide of the LNFE as can be seen in Figure 9. If the disturbance signal is applied to the connecting waveguide the limited frequency range of the C-Band waveguide shall be taken into account. Due to the ambiguous propagation modes of the used C-Band waveguide for higher frequencies, smaller waveguides with appropriate linear tapers shall be used for the measurement of higher frequencies. These frequency ranges are also referred to as waveguide bands as can be seen in Table 8.

Each waveguide band shall be measured with its corresponding waveguide resulting in unambiguously measurements for the spurious measurements.

The disturbing signal shall have the following characteristics:

the sinusoidal CW signal in the OoB domain shall increase in the same degree as the permitted emission spectrum with a limit of -90 dBpp. See Figure 4 for an example.

the sinusoidal CW signal in the spurious domain shall have a maximum amplitude of -30 dBm

the discrete frequency steps shall be equal to 1 MHz

An appropriate measurement device shall be connected to the LNFE output and shall have the following characteristics:

the frequency span shall be equal to 1 MHz

NOTE 2:	Due to the huge amount of frequency steps that shall be checked it is recommended to use a computer aided measurement system to decrease the measurement time.

The corresponding output power shall be measured at the LNFE output. This procedure will be repeated for all discrete frequency steps in the OoB and spurious domain.

After all frequency steps have been applied and its corresponding output powers have been recorded the output power levels shall be set in relation to the output power of the operating frequency.

NOTE 3:	If the limit of 90 dBpp has not been achieved at some frequencies, the output of the signal generator should be checked to see if spurious signals are present.

If spurious signals from the signal generator are present they shall be stated in the test report.

The results obtained shall be compared to the limits in clause 4.2.1.1.2. in order to prove compliance with the requirement.

# Annex A (normative): 
Relationship between the present document and the essential requirements of Directive 2014/53/EU

The present document has been produced to provide a means of conforming to the essential requirements of Directive 2014/53/EU [i.1] of the European Parliament and of the Council of 16 April 2014 on the harmonisation of the laws of the Member States relating to the making available on the market of radio equipment - also known as the Radio Equipment Directive1999/5/EC. Once the present document is cited in the Official Journal of the European Union under that Directive, compliance with the normative clauses of the present document given in Table A.1 confers, within the limits of the scope of the present document, a presumption of conformity with the corresponding essential requirements of that Directive, and associated EFTA regulations.

Table A.1: Relationship between the present document and the essential requirements
 of Directive 2014/53/EU [i.1]

| Harmonised Standard ETSI EN 303 347 The following requirements are relevant to the presumption of conformity under the article 3.2 of Directive 2014/53/EU [i.1] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement |  |  | Requirement Conditionality |  |
| No | Description | Reference: Clause No | U/C | Condition |
| 1 | Operating frequency | 4.2.1.1 | U |  |
| 2 | Out-of-Band emissions | 4.2.1.3 | U |  |
| 3 | Spurious emissions | 4.2.1.4 | U |  |
| 4 | Receiver Selectivity | 4.3.2.1 | U |  |

Key to columns:

Requirement:

No	A unique identifier for one row of the table which may be used to identify a requirement or its test specification.

Description	A textual reference to the requirement.

Clause Number	Identification of clause(s) defining the requirement in the present document unless another document is referenced explicitly.

Requirement Conditionality:

U/C	Indicates whether the requirement shall be unconditionally applicable (U) or is conditional upon the manufacturers claimed functionality of the equipment (C).

Condition	Explains the conditions when the requirement shall or shall not be applicable for a requirement which is classified "conditional".

Presumption of conformity stays valid only as long as a reference to the present document is maintained in the list published in the Official Journal of the European Union. Users of the present document should consult frequently the latest list published in the Official Journal of the European Union.

Other Union legislation may be applicable to the product(s) falling within the scope of the present document.

# Annex B (normative):
Operating frequency, transmitter power and OoB measurement setup

Figure 7: Indirect method for operating frequency and transmitter power measurement

The method for measurement of the operating frequency and the transmitter power shown in Figure 7 shall be applied.

Note:	Figure 7 shows for simplicity a single polarisation meteorological radar system. If a dual polarised system is used the single polarisation mode shall be activated. If only permanent dual polarisation mode is available and no coupler in front of the power divider is available, the coupling ratio from the power divider shall be taken into account.

# Annex C (normative):
Spurious emission measurement setup

Figure 8: Indirect method for spurious emission measurement

The coupling factor of the used couplers shall be known in the allocated frequency band with an accuracy of at least 0,5 dB or better.

Note:	Figure 8 shows for simplicity a single polarisation meteorological radar system. If a dual polarised system is used the single polarisation mode shall be activated. If only permanent dual polarisation mode is possible and no coupler in front of the power divider is available, the coupling ratio from the power divider shall be taken into account.

# Annex D (normative): 
Receiver selectivity measurement setup

Figure 9: Measurement method for receiver selectivity measurement

# Change history

| Document history |  |  |
| --- | --- | --- |

# History

| Document history |  |  |
| --- | --- | --- |
| V0.0.1 | November 2015 | Starting of draft version |
| V0.1.2 | December 2015 | - Clarification in OoB and spurious emissions conformance requirements. Added reference. - Change of operating frequency measurement procedure. |
| V0.1.3 | April 2016 | Clarification added for dual pol systems only, transmitter power measurement uncertainty changed and error corrections. |
| V0.2.4 | May 2016 | Added the description of receiver measurement with waveguide, Annex D. Minor corrections. Changed spectrum mask figures. |
| V0.2.5 | May 2016 | Changed to multipart document, added FM pulsed radar, Tx power measurement changed. |
| V0.2.6 | September 2016 | Minor technical changes, clarification of some points |
| V0.2.7 | November 2016 | Editorial changes after ERMTGAERO-RADARS#4 meeting |