Disclaimer

The present document has been produced and approved by the <long ISGname> (<short ISGname>) ETSI Industry Specification Group (ISG) and represents the views of those members who participated in this ISG.
It does not necessarily represent the views of the entire ETSI membership.

DRAFT ETSI GS NFV-SEC 020 V0.0.4(2019-06)

Group Specification

Network Function Virtualisation (NFV);

Security;

Identity Management and Security Specification

Release 3

<

Disclaimer: This DRAFT is a working document of ETSI ISG NFV. It is provided for information only and is still under development within ETSI ISG NFV. DRAFTS may be updated, deleted, replaced, or obsoleted by other documents at any time.

ETSI and its Members accept no liability for any further use/implementation of the present DRAFT.

Do not use as reference material.

Do not cite this document other than as "work in progress".

- ETSI NFV public DRAFTS are available in: http://docbox.etsi.org/ISG/NFV/Open/Drafts/

- Report FEEDBACK via the NFV issue tracker: http://nfvwiki.etsi.org/index.php?title=NFV_Issue_Tracker

- Approved and PUBLISHED deliverables shall be obtained via the ETSI Standards search page at: http://www.etsi.org/standards-search

Reference

<Workitem>

Keywords

<keywords>

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the only prevailing document is the print of the Portable Document Format (PDF) version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are trademarks of ETSI registered for the benefit of its Members.
3GPPTM and LTE™ are trademarks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
oneM2M logo is protected for the benefit of its Members.
GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	5

Foreword	5

Modal verbs terminology	5

Introduction	5

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	7

4.1 Introduction	7

4.2 Identity Definition Purposes and Uses of Identity	7

4.3 Hierarchy	8

5	Identity-Related Concepts and Definitions	9

5.1	General	9

5.1.1 TYPE and INSTANCE	9

5.1.2 Lifecycle Events	9

5.1.3 Confidentiality, Integrity, and Availability	9

5.1.4 Trust Domains	10

6	Management and Structure of Identity	11

6.1. Basic Identity Structure	11

6.2	Structure of Identity	11

6.2.1 ID Top Level Structure	11

6.2.1.1 OSS/BSS Layer Type	11

6.2.1.2 NFV Layer Type	11

6.2.1.3 OSS/BSS layer identifier	12

6.2.1.4 NFV layer identifier	12

6.3	Properties and Attributes of Identity	12

6.3.1	Purpose of Identity	12

6.3.2 Attributes bound to Identity	12

6.3.2.1 Attributes Bound to NFV TYPE	12

6.3.2.2 Attributes Bound to NFV INSTANCE	12

6.3.2.3 Behavioural Evolution and Attribute Binding	12

6.4	Purpose of Identity	12

6.5	Discovery	13

6.6	Privacy	13

7	Security constraints of identity	13

7.1	Usage and Consumption	13

7.1.1 Authentication	13

7.1.2 Authorisation	13

7.1.3 Accounting	14

7.1.4 Integrity	14

8	Identity Trust Model	14

8.1	General Model	14

8.2	Validating Trust between Multiple Domains	14

Annex A (informative): Hash Constraints	15

Annex (informative): Authors & contributors	16

Annex (informative): Bibliography	17

History	18

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

# Foreword

This Group Specification (GS) has been produced by ETSI Industry Specification Group (ISG) Network Functions Virtualisation (NFV).

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Introduction

# 1	Scope

The present document specifies requirements for secure VNF identity management and trust relationships in NFV. The present document specifies how identities are securely lifecycle managed, verified and trusted. The present document addresses both horizontal and vertical relationships and leverages existing work in NFV SEC 005[5], NFV SEC 007[4], NFV SEC 009[3], NFV SEC 012[1] and NFV SEC 013[2].

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	ETSI GS SEC 012 "Network Functions Virtualisation (NFV); NFV Security; System architecture for execution of sensitive NFV components – specification"

[2]	ETSI GS SEC 013 "Network Functions Virtualisation (NFV); Security Management and Monitoring specification"

[3]	ETSI GS SEC 009 "Network Functions Virtualization (NFV); Security; Report on use cases and technical approaches for multi-layer host administration"

[4]		ETSI GR NFV-SEC 007 "Network Function Virtualisation (NFV); Trust; Report on Attestation Technologies and Practices for Secure Deployments"

[5]	ETSI GR NFV-SEC 005 "Network Functions Virtualization (NFV); Trust; Report on Certificate Management"

.

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	<Standard Organization acronym> <document number><version number/date of publication>: "<Title>".

[i.2]	etc.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

## 3.2	Symbols

## 3.3	Abbreviations

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

# 4	Overview

## 4.1 Introduction

Everything that makes up a telecoms network and everything or person who uses the network needs identity (ID). It is the fact of being as to who or what a person or thing is and determines the characteristics of the same. For CSPs, identity of NFVI components, SDN routing and VNFs are the key to how CSPs design, manage and operate their networks.

Identities may be self-assigned, given, inherited, derived, acquired, allocated or obtained in a large number of ways.

If an attacker obtains access to an NFV implemented CSP network then it must be possible months after the event to retrace the attack to establish where they got into the network, what was accessed, for how long and as far as possible, what identity they used to achieve this access. Similarly, if a customer reports a fault it must be possible to trace their current and past usage of services to resolve the issue.

Therefore, in an NFV environment it must be possible for identities to be trusted, structured, unique and immutable for a given period, if networks are to be operated securely and with a low risk of fraud.

The present document describes secure identity management in the context of NFV, in terms of what an identity is, what that identity is used for, how it is assigned, how it is discovered and how it is securely managed throughout the lifecycle of that identity.

## 4.2 Identity Definition Purposes and Uses of Identity

The present document defines an ID structure in clause that vertically spans the NFV domain and the application domain above it. Information from both domains is necessary to implement real-world, effective security policies. The ID will contain information about both the TYPE and INSTANCE of a software process. The information about the TYPE of a running INSTANCE available in a higher trust domain shall not be available in any lower trust domain. Further, information about the TYPE of a running INSTANCE available in any trust domain shall not be available in any trust domain of equal sensitivity, unless the process in the source trust domains explicitly intends it, as expressed in appropriate security policies.

Identity is the lowest brick in the foundation of networks. It enables distinction among individual instances and individual types, discovery of suitable partners of a process, attachment to such partners once discovered, and as the lowest brick in the foundation of security, identity enables the assignment, tracking and evolution of trust.

Figure 4-1. Purposes and Uses of Identity

There are two basic flavours of identity in a network: type identity, through which architecture is structured, and instance identity, through which the runtime is structured.

Identity enables the development of ontologies, allowing the systems designer to make statements about an object’s capabilities and uses, and based on this, allows the implementer to distinguish, manage and secure runtime instantiations of said objects.

## 4.3 Hierarchy

IDs have usage and meaning that span domains. Figure 4-1 depicts the main elements that share usage and meaning of IDs in the larger context in which NFV exists. While the figure shows the relationships, the details of the actual interfaces themselves are outside the scope of the document.

Figure 4-2. NFV Context

Operations Support Systems (OSS) and Business Support Systems (BSS) have existed and will exist outside the NFV domain, but will continue to be intricately linked with the functioning of virtualized functions. As will become clear from the rest of the present document, some interaction, whether automated or manual, will have to exist between OSS/BSS and the application layer that uses NFV as its infrastructure, and, respectively between OSS/BSS and the Security Manager (SM) NFV element, and MANO.

# 5	Identity-Related Concepts and Definitions

## 5.1	General

### 5.1.1 TYPE and INSTANCE

The present document defines a software package TYPE category, which describes the particular functions the package is capable of fulfilling at the application layer (such as a firewall, a 3GPP-defined Serving Call Session Control Function [S-CSCF], etc.), and the INSTANCE, which identifies the running instance of the VNF and/or its component software process.

### 5.1.2 Lifecycle Events

Further, the present document defines two events in the lifetime of a software package: ON-BOARDING time, when the software package is received from the vendor and is added to the software catalog of the CSP, and run-time events INSTANTIATION, MODIFICATION, and TERMINATION, when the VNFI is executed, modified, and terminated, respectively.

Editor’s Note: There may exist special constraints associated with the event of instantiating the software package (or members of the package) for the first time, and these will be pointed out when relevant.

Editor’s Note: If some of the defined events and concepts are superfluous, they will be removed before publication. They are here now for completeness’ sake only.

### 5.1.3 Confidentiality, Integrity, and Availability

The US National Institute of Standards and Technology (NIST) defines the Confidentiality/Integrity/Availability (CIA) model [REF: https://nvlpubs.nist.gov/nistpubs/FIPS/NIST.FIPS.199.pdf], which defines LOW/MODERATE/HIGH impact to each of the three dimensions. We adopt this model for the present document. The attributes can be applied at both ON-BOARDING time, to the TYPE of a package, and at INSTANTIATION time, to the running INSTANCE of the package.

Figure 5-1. CIA Model

For the purposes of this document, we define the same three levels, LOW, MODERATE, and HIGH, for the three dimensions. While all three dimensions play into ID management, we concentrate on the Confidentiality dimension. At the discretion of the CSP, the NFV system shall operate with at least two of the three confidentiality levels, with the Security Manager assigned to the higher confidentiality level of the two, and MANO assigned to the lower level of the two. If the CSP elects to implement all three levels, the SM shall be assigned to the HIGH confidentiality level, and MANO shall be assigned to the LOW confidentiality level.

For the purposes of the present document, the Availability dimension is expanded to encompass Availability itself, Authorization, and Authentication. Again, the same three levels are defined for these.

### 5.1.4 Trust Domains

A trust domain is defined as a set of processes running at the same sensitivity level, as a result of the application of a common set of security policies. The CSP shall manage the flow of information across trust domain in such a way that information and attributes in a higher sensitivity trust domain shall not transfer to a lower sensitivity trust domain.

There may exist many separate trust domains of the same sensitivity level, but information contain in each is not necessarily available from any other. For example, administrators given access to the HIGH sensitivity level will not necessarily have access to other trust domain of HIGH sensitivity level, as dictated by the need-to-know principle. In the rest of the present document we may refer to a “HIGH trust zone,” and it shall always be read to mean a HIGH confidentiality trust zone, unless the other two attributes are used explicitly. A consequence of this requirement is that data needs to be labelled as exportable or non-exportable.

Implementing an access control system shall be mandatory. As described in SEC-012 [1], maintaining security is more fine grained under the implementation of Attribute-Based Access Control (ABAC), rather than the simpler Role-Based Access Control (RBAC). A difference between the two is that ABAC also takes into consideration the context (e.g., time of day access restrictions to certain resources) of a resource access event, not only the accessor and the resource itself. ABAC is comparable to concepts used in multi-factor authentication, RBAC is comparable to concepts used in single-factor authentication. Segregating access to a trust domain is therefore more robust under ABAC.

# 6	Management and Structure of Identity

## 6.1. Basic Identity Structure

There are two events in the CSP NFV network that generate a need to fix identifiers: onboarding time, and instantiation time. The onboarding event fixes the TYPE of an object, which cannot change at instantiation time (or at any time after onboarding). The instantiation event fixes the INSTANCE of an object, which remains the specified TYPE for the lifetime of the INSTANCE. Therefore, in its most basic form, the NFV identifier has two parts INSTANCE and TYPE.

There are two strategies that can be employed to define IDs in NFV:

- Generate standard unique identifiers for both the TYPE and INSTANCE parts, then bind information from other sources (such as the VNFD) to the ID cryptographically, hashing the combination and keeping the resulting hashes in the runtime management system, or

- Create a standard structure inside the TYPE and INSTANCE such that information can be gleaned directly from the ID, without the need to look up another source, (e.g., dedicate the first few bits in the TYPE part to the manufacturer ID, akin to the fashion in which the IEEE manages Organizationally Unique Identifiers [OUIs] and Company IDs [CIDs]), and, obviously, cryptographically protect the IDs as in 1).

The first variant holds a clear flexibility advantage: the attributes bound to an ID can vary across CSPs, across internal CSP divisions, etc. However, interworking across CSPs is hamstrung.

The second variant is more heavily subject to standardization. If the second variant is chosen, an NFV Registration Authority (NRA) that maintains number assignments must be instituted. That will require an economic ecosystem with incentives that ensure the task is managed in perpetuity [Editor’s Note: link to new Permissioned Distributed Ledger ISG work product – ask Diego].

ID structure shall follow method 2), with TYPE structure pre-defined, and INSTANCE ID generated as standard unique identifiers.

## 6.2	Structure of Identity

### 6.2.1 ID Top Level Structure

There exist exactly two top level components to an identity in a CSP NFV network: an identity for the TYPE of the object, and one for the running INSTANCE. They have different properties, are assigned at different stages, by different entities, and have different security requirements.

| TYPE | INSTANCE |
| --- | --- |

Table 6-1. ID Structure

There are two interacting layers that identity management needs to support: the NFV layer and the OSS/BSS layer:

| SS/BSS layer TYPE | NFV layer TYPE | OSS/BSS layer IDENTIFIER | NFV layer IDENTIFIER |
| --- | --- | --- | --- |

Table 6-2. Cross-Layer Structure

#### 6.2.1.1 OSS/BSS Layer Type

This field is reserved for the use of the OSS/BSS layer, and is meant to be defined in other standards bodies, at the application layer from the NFV point of view (e.g., 3GPP types such as AMF, UDM, PCF, S-CSCF, P-CSCF). The management of that space is the responsibility of the application layer standards body.

#### 6.2.1.2 NFV Layer Type

This field is used by NFV, and its structure can be further defined in the scope of MANO.

#### 6.2.1.3 OSS/BSS layer identifier

This identifier is generated by the OSS/BSS, and is out of scope of NFV as far as its format and generation go, but NFV shall not decouple it from this four element identity structure.

#### 6.2.1.4 NFV layer identifier

This identifier is used by NFV, and its generation and management is in the scope of MANO.

## 6.3	Properties and Attributes of Identity

Editor’s Note: Topics to include:

Roles and Groups.

Atomic across the system.

Persistence?

Identity is attached to the VM state.?

Are there relationships between the physical and logical layer.

- Does this create dependencies between the two.

Needs to be able to support migration

Hardware Catalogue?

Mapping of VNF IDs to physical hardware IDs (both directions).

## 6.3.1	Purpose of Identity

A mechanism to ensure uniqueness of identity across the CSP system at any given time shall be employed.

### 6.3.2 Attributes bound to Identity

#### 6.3.2.1 Attributes Bound to NFV TYPE

The following are attributes that are not part of the ID itself, but shall nevertheless be able to be cryptographically bound to the TYPE part by the Security Manager, fixed upon onboarding a package received from the vendor into the CSP catalogue:

- Manufacturer

- Version

#### 6.3.2.2 Attributes Bound to NFV INSTANCE

The following are attributes that are not part of the ID itself, but shall nevertheless be able to be cryptographically bound to the INSTANCE part by the Security Manager, fixed upon the completion of the launch procedure of an instance of a VNF:

- TYPE

- Timestamp

- Locstamp (e.g., geolocation of data centre, host ID)

- Level Of Assurance (1,2,3,4,5a,5b – see REF: ETSI GR NFV-SEC 007 V1.1.1 (2017-10), Section 5)

- Identification of MANO function instances (e.g. NFVO, VNFM, VIM) which effected the launch

#### 6.3.2.3 Behavioural Evolution and Attribute Binding

As the NFV system is used, it evolves, and new information is gathered and created that may change the setting or meaning of attributes bound at ON-BOARDING time or INSTANTIATION time. Once a TYPE ID or INSTANCE ID is fixed, the SM shall not change them. However, the values of the attributes bound to the ID may change, and the SM may modify the bound attributes, but each SM, within its trust domain, shall make a clear, atomic, immutable but appendable, cryptographically bound log entry for every change. These SM logs shall be available for auditing at least for the lifetime of the longest lived element managed by the SM, or longer, as dictated by legal considerations or CSP policy.

## 6.4	Purpose of Identity

Purpose of identity

- Distinction

- Discovery

- Attachment

- Trust

- Authorisation of VM/ VNFCI or VNFI to do things?

  - Assert security associations to other VMs or VNFIs.

- Attest to identity being the correct VM or VNFI.

- Assert to overall state of the network as the sum of the state of all IDs.

- Binding of VM and VNFIs to hardware & Hypervisor (Entire stack downwards).

- Linkage to Legacy Physical network elements

- Discovery

VIM, MANO and associated identities.

[Editor’s Note: the above pretty much covers it, it only needs to be converted to text – sorry – ran out of time]

## 6.5	Discovery

How is are IDs discovered?

Clairvoyance?

Have four-fold structure that allows mapping from one layer to another.

## 6.6	Privacy

- Need to separate personal privacy from package/artefact confidentiality.

- When a slice/service/function can be mapped to a person, GDPR concerns apply.

- When data in a slice/service/function is stored and contains (Sensitive) Personally Identifiable Information ([S]PII), GDPR concerns apply

- Consent

- Multi-tenant consideration – stop one tenant from accessing other tenants/infrastructure provider’s PII

- Which if any elements of an NFV ID could result in PII. Type may be guide to existence of PII concerns.

# 7	Security constraints of identity

Long ID permanent requirements

Short ID term requirements.

Replay prevention

Uniqueness

Lifecycle

Temp vs long term ID Entropy / predictability vs appropriate security mechanisms.

External ID visibility vs internal visibility (eg ID@realm)

Destruction or removal of ID

Right to be forgotten?

Roles and Groups.

Assertion and assurance

## 7.1	Usage and Consumption

### 7.1.1 Authentication

The administrator that performs the on-boarding step shall be authenticated before each onboarding action, i.e., there will be a one-to-one relationship between an administrative login and a package on-boarding action.

### 7.1.2 Authorisation

The administrator that performs the on-boarding step shall be specifically authorized to do so.

### 7.1.3 Accounting

The Security Manager shall maintain logs of all operations performed, who performed them, and where (infrastructure/host).

### 7.1.4 Integrity

# 8	Identity Trust Model

## 8.1	General Model

## 8.2	Validating Trust between Multiple Domains

How do requirements in 5 & 6 apply across multiple domains.

TODO: Expand on the SM-LI relationship at RUN-TIME.

TODO: Includes multi-vendor environments. IMPORTANT!

At a minimum insert some motherhood and apple pie statement and point to SEC013.

Annex A (informative):
Hash Constraints

The main reason to consider hashing efficiency is the security need to hide actual IDs and perform network management as much as possible on hashes of real IDs. SHA-2-256 is faster on fast-disappearing 32-bit hardware, due to the 32-bit internal state chunks, while SHA-2-512 is more efficient on modern 64-bit hardware, for any length input. Therefore, 512 bits is a sweet spot. The question now becomes, for how long?

Gartner [REF: https://www.gartner.com/imagesrv/books/iot/iotEbook_digital.pdf] predicts twenty billion Internet of Things (IoT) devices will be in use by 2020.  An identifier size of 35 bits (2^35 = 34,359,738,368) will cover that. We can expect the number of VNFs to be at least two orders of magnitude below this, therefore 29 bits should suffice for the year 2020. In conclusion, an INSTANCE ID size of 128 bits (a space of 3.4 x 1038) should be sufficient for a good long time to cover the space.

The next requirement to consider is collision resistance, and pre-image resistance. NIST publishes the following table [REF: https://nvlpubs.nist.gov/nistpubs/FIPS/NIST.FIPS.202.pdf] that characterizes the security strength of various hashing functions:

Table A-1. NIST Security Strength Table

Collision resistance is easy to define: it is the probability that there exist any two objects that have the same hash. Pre-image collisions are a special type of collision, one in which an object is chosen, then another object is sought such that it has the same hash as the chosen one. It is intuitive that the second proposition is harder, or probabilistically less likely. As expected, the table follows this pattern.

“Security strength,” again, according to NIST, is “a number associated with the amount of work (that is, the number of operations of some sort) that is required to break a cryptographic algorithm or system in some way. […] If the security strength associated with an algorithm or system is S bits, then it is expected that (roughly) 2S basic operations are required to break it.” [REF: https://nvlpubs.nist.gov/nistpubs/SpecialPublications/NIST.SP.800-90Ar1.pdf].

A design choice is made therefore to achieve a minimum of 128 bits of collision resistance, and 256 bits of pre-image resistance. SHA-384 satisfies both, with 192 bits of collision resistance and 384 bits of pre-image resistance. The only reason not to use 512, is to use the remaining 128 bits (512 - 384) for the TYPE part of the ID. This results in a total size of 512 bits, which, as stated before, is perfectly fit to hash on the 64-bit machines of today.

Annex (informative):
Authors & contributors

The following people have contributed to the present document:

Rapporteur:
Mr Leslie Willis, BT Group Plc (UK)

Other contributors:
Mr Michael Bilca, OTD

Mr Alex Leadbeater, BT Group Plc (UK)

Annex (informative):
Bibliography

# History

| Document history |  |  |
| --- | --- | --- |
| <Version> | <Date> | <Milestone> |
| 0.0.1 | 05/12/2017 | First draft based on NFVSEC(17)000166r1 |
| 0.0.1a | 17/05/2018 | Output of drafting session at NFV#22 / NFV SEC#124 F2F |
| 0.0.2 | 20/09/2018 | Output to NFV#23 / NFV SEC#131 F2F |
| 0.0.3 | 22/02/2019 | Output of NFV#25 / NFV SEC#141 Beijing meeting |
| 0.0.4 | 23/05/2019 | Output of NFV#26 / NFV SEC#147 Sophia Antipolis meeting |